Кратко описание на програмата за управление на огледало

Тази програма е разработена за микроконтролер STM32 и реализира управление на огледало в две оси – хоризонтална (ляво/дясно) и вертикална (нагоре/надолу) чрез джойстик. Използват се два аналогово-цифрови преобразувателя (ADC), които четат аналоговите сигнали от обратната връзка и ги преобразуват в цифрови стойности.

В зависимост от посоката на отклонение на джойстика, програмата управлява изходни GPIO пинове, които задават посоката на движение на огледалото. PWM сигнал, генериран чрез таймер TIM3, се използва за управление на интезитета на мигача.

Когато аналоговата стойност от ADC достигне предварително зададени гранични стойности, движението в съответната посока се прекратява, което осигурява ограничаване на хода на огледалото. В края на всеки цикъл всички изходи се нулират, за да се гарантира безопасна работа на системата.


(Конфигурация в cubemx)

(снимка на опитна постанопвка)

(Платка за управление)



















сорс код:
/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file : main.c

* @brief : Main program body

******************************************************************************

* @attention

*

* Copyright (c) 2025 STMicroelectronics.

* All rights reserved.

*

* This software is licensed under terms that can be found in the LICENSE file

* in the root directory of this software component.

* If no LICENSE file comes with this software, it is provided AS-IS.

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

ADC_HandleTypeDef hadc1;

ADC_HandleTypeDef hadc2;

COMP_HandleTypeDef hcomp2;

COMP_HandleTypeDef hcomp3;

COMP_HandleTypeDef hcomp4;

COMP_HandleTypeDef hcomp6;

DAC_HandleTypeDef hdac1;

DAC_HandleTypeDef hdac2;

DAC_HandleTypeDef hdac3;

HRTIM_HandleTypeDef hhrtim1;

TIM_HandleTypeDef htim3;

UART_HandleTypeDef huart3;

PCD_HandleTypeDef hpcd_USB_FS;

/* USER CODE BEGIN PV */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_ADC1_Init(void);

static void MX_COMP2_Init(void);

static void MX_COMP3_Init(void);

static void MX_COMP4_Init(void);

static void MX_COMP6_Init(void);

static void MX_DAC1_Init(void);

static void MX_DAC2_Init(void);

static void MX_DAC3_Init(void);

static void MX_HRTIM1_Init(void);

static void MX_TIM3_Init(void);

static void MX_UCPD1_Init(void);

static void MX_USART3_UART_Init(void);

static void MX_USB_PCD_Init(void);

static void MX_ADC2_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

unit16_t ADC_UP/Down

unit16_t ADC_Left/Right

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_ADC1_Init();

MX_COMP2_Init();

MX_COMP3_Init();

MX_COMP4_Init();

MX_COMP6_Init();

MX_DAC1_Init();

MX_DAC2_Init();

MX_DAC3_Init();

MX_HRTIM1_Init();

MX_TIM3_Init();

MX_UCPD1_Init();

MX_USART3_UART_Init();

MX_USB_PCD_Init();

MX_ADC2_Init();

/* USER CODE BEGIN 2 */

// HAL_TIM_PWM_Start(&htim3, TIM_CHANNEL_4);

HAL_ADCEx_Calibration_Start(&hadc1, ADC_SINGLE_ENDED);

HAL_ADCEx_Calibration_Start(&hadc2, ADC_SINGLE_ENDED);

HAL_ADC_Start_IT(&hadc1);

HAL_ADC_Start_IT(&hadc2);

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

while(!HAL_GPIO_ReadPin(GPIOC, GPIO_PIN_13))// while the sel button is pressed start PWM 50% and stop for half a second

{

HAL_Delay(1);

HAL_TIM_PWM_Start(&htim3, TIM_CHANNEL_4);

HAL_Delay(500);

HAL_TIM_PWM_Stop(&htim3, TIM_CHANNEL_4);

HAL_Delay(500);

}

while(!HAL_GPIO_ReadPin(GPIOC, GPIO_PIN_4))//check if joystick left

{

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_1, 1);//SEL = 1

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4, 1);//go left

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_0, 0);//PWM = 1

HAL_ADC_Start(&hadc2);

HAL_ADC_PollForConversion(&hadc1, 200);

ADC_Left/Right = HAL_ADC_GetValue(&hadc2);

HAL_ADC_Stop(&hadc2);

HAL_Delay(1);

HAL_TIM_Base_Start(&htim1);

if(ADC_Left/Right < 10){

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4, 0);

break;

}

}

while(!HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_2))//check if joystick right

{

HAL_ADC_Start(&hadc2);

HAL_ADC_PollForConversion(&hadc1, 200);

ADC_Left/Right = HAL_ADC_GetValue(&hadc2);

HAL_ADC_Stop(&hadc2);

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_1, 1);// SEL 1

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, 1);//go right

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_0, 0);//PWM 1

HAL_Delay(1);

HAL_TIM_Base_Start(&htim1);

if(ADC_Left/Right > 5000){

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, 0);

break;

}

}

while(!HAL_GPIO_ReadPin(GPIOC, GPIO_PIN_5))//check if joystick down

{

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_0, 1);//go down

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_8, 1);//sel 1

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_5, 1);//pwm 1

HAL_ADC_Start(&hadc1);

HAL_ADC_PollForConversion(&hadc1, 200);

ADC_UP/Down = HAL_ADC_GetValue(&hadc1);

HAL_ADC_Stop(&hadc1);

HAL_Delay(1);

HAL_TIM_Base_Start(&htim1);

if(ADC_UP/Down < 10){

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_0, 0);

break;

}

}

while(!HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_10))//check if joystick up

{

HAL_ADC_Start(&hadc1);

HAL_ADC_PollForConversion(&hadc1, 200);

ADC_UP/Down = HAL_ADC_GetValue(&hadc1);

HAL_ADC_Stop(&hadc1);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_8, 1);//sel 1

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_5, 1);//pwm 1

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_9, 1);//go up

HAL_Delay(1);

HAL_TIM_Base_Start(&htim1);

if(ADC_UP/Down > 5000){

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_9, 0);

break;

}

}

//zero all outputs

HAL_TIM_Base_Stop(&htim1);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4, 0);

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, 0);

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_0, 0);

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_1, 0);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_8, 0);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_9, 0);

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_1, 0);

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_0, 0);

HAL_TIM_PWM_Stop(&htim3, TIM_CHANNEL_4);

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

HAL_PWREx_ControlVoltageScaling(PWR_REGULATOR_VOLTAGE_SCALE1_BOOST);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI|RCC_OSCILLATORTYPE_HSI48;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.HSI48State = RCC_HSI48_ON;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

RCC_OscInitStruct.PLL.PLLM = RCC_PLLM_DIV4;

RCC_OscInitStruct.PLL.PLLN = 85;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV8;

RCC_OscInitStruct.PLL.PLLQ = RCC_PLLQ_DIV8;

RCC_OscInitStruct.PLL.PLLR = RCC_PLLR_DIV2;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_4) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief ADC1 Initialization Function

* @param None

* @retval None

*/

static void MX_ADC1_Init(void)

{

/* USER CODE BEGIN ADC1_Init 0 */

/* USER CODE END ADC1_Init 0 */

ADC_MultiModeTypeDef multimode = {0};

ADC_ChannelConfTypeDef sConfig = {0};

/* USER CODE BEGIN ADC1_Init 1 */

/* USER CODE END ADC1_Init 1 */

/** Common config

*/

hadc1.Instance = ADC1;

hadc1.Init.ClockPrescaler = ADC_CLOCK_SYNC_PCLK_DIV4;

hadc1.Init.Resolution = ADC_RESOLUTION_12B;

hadc1.Init.DataAlign = ADC_DATAALIGN_RIGHT;

hadc1.Init.GainCompensation = 0;

hadc1.Init.ScanConvMode = ADC_SCAN_DISABLE;

hadc1.Init.EOCSelection = ADC_EOC_SEQ_CONV;

hadc1.Init.LowPowerAutoWait = DISABLE;

hadc1.Init.ContinuousConvMode = DISABLE;

hadc1.Init.NbrOfConversion = 1;

hadc1.Init.DiscontinuousConvMode = DISABLE;

hadc1.Init.ExternalTrigConv = ADC_SOFTWARE_START;

hadc1.Init.ExternalTrigConvEdge = ADC_EXTERNALTRIGCONVEDGE_NONE;

hadc1.Init.DMAContinuousRequests = DISABLE;

hadc1.Init.Overrun = ADC_OVR_DATA_PRESERVED;

hadc1.Init.OversamplingMode = DISABLE;

if (HAL_ADC_Init(&hadc1) != HAL_OK)

{

Error_Handler();

}

/** Configure the ADC multi-mode

*/

multimode.Mode = ADC_MODE_INDEPENDENT;

if (HAL_ADCEx_MultiModeConfigChannel(&hadc1, &multimode) != HAL_OK)

{

Error_Handler();

}

/** Configure Regular Channel

*/

sConfig.Channel = ADC_CHANNEL_8;

sConfig.Rank = ADC_REGULAR_RANK_1;

sConfig.SamplingTime = ADC_SAMPLETIME_2CYCLES_5;

sConfig.SingleDiff = ADC_SINGLE_ENDED;

sConfig.OffsetNumber = ADC_OFFSET_NONE;

sConfig.Offset = 0;

if (HAL_ADC_ConfigChannel(&hadc1, &sConfig) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN ADC1_Init 2 */

/* USER CODE END ADC1_Init 2 */

}

/**

* @brief ADC2 Initialization Function

* @param None

* @retval None

*/

static void MX_ADC2_Init(void)

{

/* USER CODE BEGIN ADC2_Init 0 */

/* USER CODE END ADC2_Init 0 */

ADC_ChannelConfTypeDef sConfig = {0};

/* USER CODE BEGIN ADC2_Init 1 */

/* USER CODE END ADC2_Init 1 */

/** Common config

*/

hadc2.Instance = ADC2;

hadc2.Init.ClockPrescaler = ADC_CLOCK_SYNC_PCLK_DIV4;

hadc2.Init.Resolution = ADC_RESOLUTION_12B;

hadc2.Init.DataAlign = ADC_DATAALIGN_RIGHT;

hadc2.Init.GainCompensation = 0;

hadc2.Init.ScanConvMode = ADC_SCAN_DISABLE;

hadc2.Init.EOCSelection = ADC_EOC_SINGLE_CONV;

hadc2.Init.LowPowerAutoWait = DISABLE;

hadc2.Init.ContinuousConvMode = DISABLE;

hadc2.Init.NbrOfConversion = 1;

hadc2.Init.DiscontinuousConvMode = DISABLE;

hadc2.Init.ExternalTrigConv = ADC_SOFTWARE_START;

hadc2.Init.ExternalTrigConvEdge = ADC_EXTERNALTRIGCONVEDGE_NONE;

hadc2.Init.DMAContinuousRequests = DISABLE;

hadc2.Init.Overrun = ADC_OVR_DATA_PRESERVED;

hadc2.Init.OversamplingMode = DISABLE;

if (HAL_ADC_Init(&hadc2) != HAL_OK)

{

Error_Handler();

}

/** Configure Regular Channel

*/

sConfig.Channel = ADC_CHANNEL_3;

sConfig.Rank = ADC_REGULAR_RANK_1;

sConfig.SamplingTime = ADC_SAMPLETIME_2CYCLES_5;

sConfig.SingleDiff = ADC_SINGLE_ENDED;

sConfig.OffsetNumber = ADC_OFFSET_NONE;

sConfig.Offset = 0;

if (HAL_ADC_ConfigChannel(&hadc2, &sConfig) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN ADC2_Init 2 */

/* USER CODE END ADC2_Init 2 */

}

/**

* @brief COMP2 Initialization Function

* @param None

* @retval None

*/

static void MX_COMP2_Init(void)

{

/* USER CODE BEGIN COMP2_Init 0 */

/* USER CODE END COMP2_Init 0 */

/* USER CODE BEGIN COMP2_Init 1 */

/* USER CODE END COMP2_Init 1 */

hcomp2.Instance = COMP2;

hcomp2.Init.InputPlus = COMP_INPUT_PLUS_IO1;

hcomp2.Init.InputMinus = COMP_INPUT_MINUS_DAC3_CH2;

hcomp2.Init.OutputPol = COMP_OUTPUTPOL_NONINVERTED;

hcomp2.Init.Hysteresis = COMP_HYSTERESIS_NONE;

hcomp2.Init.BlankingSrce = COMP_BLANKINGSRC_NONE;

hcomp2.Init.TriggerMode = COMP_TRIGGERMODE_NONE;

if (HAL_COMP_Init(&hcomp2) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN COMP2_Init 2 */

/* USER CODE END COMP2_Init 2 */

}

/**

* @brief COMP3 Initialization Function

* @param None

* @retval None

*/

static void MX_COMP3_Init(void)

{

/* USER CODE BEGIN COMP3_Init 0 */

/* USER CODE END COMP3_Init 0 */

/* USER CODE BEGIN COMP3_Init 1 */

/* USER CODE END COMP3_Init 1 */

hcomp3.Instance = COMP3;

hcomp3.Init.InputPlus = COMP_INPUT_PLUS_IO1;

hcomp3.Init.InputMinus = COMP_INPUT_MINUS_DAC3_CH1;

hcomp3.Init.OutputPol = COMP_OUTPUTPOL_NONINVERTED;

hcomp3.Init.Hysteresis = COMP_HYSTERESIS_NONE;

hcomp3.Init.BlankingSrce = COMP_BLANKINGSRC_NONE;

hcomp3.Init.TriggerMode = COMP_TRIGGERMODE_NONE;

if (HAL_COMP_Init(&hcomp3) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN COMP3_Init 2 */

/* USER CODE END COMP3_Init 2 */

}

/**

* @brief COMP4 Initialization Function

* @param None

* @retval None

*/

static void MX_COMP4_Init(void)

{

/* USER CODE BEGIN COMP4_Init 0 */

/* USER CODE END COMP4_Init 0 */

/* USER CODE BEGIN COMP4_Init 1 */

/* USER CODE END COMP4_Init 1 */

hcomp4.Instance = COMP4;

hcomp4.Init.InputPlus = COMP_INPUT_PLUS_IO1;

hcomp4.Init.InputMinus = COMP_INPUT_MINUS_DAC1_CH1;

hcomp4.Init.OutputPol = COMP_OUTPUTPOL_NONINVERTED;

hcomp4.Init.Hysteresis = COMP_HYSTERESIS_NONE;

hcomp4.Init.BlankingSrce = COMP_BLANKINGSRC_NONE;

hcomp4.Init.TriggerMode = COMP_TRIGGERMODE_NONE;

if (HAL_COMP_Init(&hcomp4) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN COMP4_Init 2 */

/* USER CODE END COMP4_Init 2 */

}

/**

* @brief COMP6 Initialization Function

* @param None

* @retval None

*/

static void MX_COMP6_Init(void)

{

/* USER CODE BEGIN COMP6_Init 0 */

/* USER CODE END COMP6_Init 0 */

/* USER CODE BEGIN COMP6_Init 1 */

/* USER CODE END COMP6_Init 1 */

hcomp6.Instance = COMP6;

hcomp6.Init.InputPlus = COMP_INPUT_PLUS_IO1;

hcomp6.Init.InputMinus = COMP_INPUT_MINUS_DAC2_CH1;

hcomp6.Init.OutputPol = COMP_OUTPUTPOL_NONINVERTED;

hcomp6.Init.Hysteresis = COMP_HYSTERESIS_NONE;

hcomp6.Init.BlankingSrce = COMP_BLANKINGSRC_NONE;

hcomp6.Init.TriggerMode = COMP_TRIGGERMODE_NONE;

if (HAL_COMP_Init(&hcomp6) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN COMP6_Init 2 */

/* USER CODE END COMP6_Init 2 */

}

/**

* @brief DAC1 Initialization Function

* @param None

* @retval None

*/

static void MX_DAC1_Init(void)

{

/* USER CODE BEGIN DAC1_Init 0 */

/* USER CODE END DAC1_Init 0 */

DAC_ChannelConfTypeDef sConfig = {0};

/* USER CODE BEGIN DAC1_Init 1 */

/* USER CODE END DAC1_Init 1 */

/** DAC Initialization

*/

hdac1.Instance = DAC1;

if (HAL_DAC_Init(&hdac1) != HAL_OK)

{

Error_Handler();

}

/** DAC channel OUT1 config

*/

sConfig.DAC_HighFrequency = DAC_HIGH_FREQUENCY_INTERFACE_MODE_AUTOMATIC;

sConfig.DAC_DMADoubleDataMode = DISABLE;

sConfig.DAC_SignedFormat = DISABLE;

sConfig.DAC_SampleAndHold = DAC_SAMPLEANDHOLD_DISABLE;

sConfig.DAC_Trigger = DAC_TRIGGER_NONE;

sConfig.DAC_Trigger2 = DAC_TRIGGER_NONE;

sConfig.DAC_OutputBuffer = DAC_OUTPUTBUFFER_DISABLE;

sConfig.DAC_ConnectOnChipPeripheral = DAC_CHIPCONNECT_INTERNAL;

sConfig.DAC_UserTrimming = DAC_TRIMMING_FACTORY;

if (HAL_DAC_ConfigChannel(&hdac1, &sConfig, DAC_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN DAC1_Init 2 */

/* USER CODE END DAC1_Init 2 */

}

/**

* @brief DAC2 Initialization Function

* @param None

* @retval None

*/

static void MX_DAC2_Init(void)

{

/* USER CODE BEGIN DAC2_Init 0 */

/* USER CODE END DAC2_Init 0 */

DAC_ChannelConfTypeDef sConfig = {0};

/* USER CODE BEGIN DAC2_Init 1 */

/* USER CODE END DAC2_Init 1 */

/** DAC Initialization

*/

hdac2.Instance = DAC2;

if (HAL_DAC_Init(&hdac2) != HAL_OK)

{

Error_Handler();

}

/** DAC channel OUT1 config

*/

sConfig.DAC_HighFrequency = DAC_HIGH_FREQUENCY_INTERFACE_MODE_AUTOMATIC;

sConfig.DAC_DMADoubleDataMode = DISABLE;

sConfig.DAC_SignedFormat = DISABLE;

sConfig.DAC_SampleAndHold = DAC_SAMPLEANDHOLD_DISABLE;

sConfig.DAC_Trigger = DAC_TRIGGER_NONE;

sConfig.DAC_Trigger2 = DAC_TRIGGER_NONE;

sConfig.DAC_OutputBuffer = DAC_OUTPUTBUFFER_DISABLE;

sConfig.DAC_ConnectOnChipPeripheral = DAC_CHIPCONNECT_INTERNAL;

sConfig.DAC_UserTrimming = DAC_TRIMMING_FACTORY;

if (HAL_DAC_ConfigChannel(&hdac2, &sConfig, DAC_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN DAC2_Init 2 */

/* USER CODE END DAC2_Init 2 */

}

/**

* @brief DAC3 Initialization Function

* @param None

* @retval None

*/

static void MX_DAC3_Init(void)

{

/* USER CODE BEGIN DAC3_Init 0 */

/* USER CODE END DAC3_Init 0 */

DAC_ChannelConfTypeDef sConfig = {0};

/* USER CODE BEGIN DAC3_Init 1 */

/* USER CODE END DAC3_Init 1 */

/** DAC Initialization

*/

hdac3.Instance = DAC3;

if (HAL_DAC_Init(&hdac3) != HAL_OK)

{

Error_Handler();

}

/** DAC channel OUT1 config

*/

sConfig.DAC_HighFrequency = DAC_HIGH_FREQUENCY_INTERFACE_MODE_AUTOMATIC;

sConfig.DAC_DMADoubleDataMode = DISABLE;

sConfig.DAC_SignedFormat = DISABLE;

sConfig.DAC_SampleAndHold = DAC_SAMPLEANDHOLD_DISABLE;

sConfig.DAC_Trigger = DAC_TRIGGER_NONE;

sConfig.DAC_Trigger2 = DAC_TRIGGER_NONE;

sConfig.DAC_OutputBuffer = DAC_OUTPUTBUFFER_DISABLE;

sConfig.DAC_ConnectOnChipPeripheral = DAC_CHIPCONNECT_INTERNAL;

sConfig.DAC_UserTrimming = DAC_TRIMMING_FACTORY;

if (HAL_DAC_ConfigChannel(&hdac3, &sConfig, DAC_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

/** DAC channel OUT2 config

*/

if (HAL_DAC_ConfigChannel(&hdac3, &sConfig, DAC_CHANNEL_2) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN DAC3_Init 2 */

/* USER CODE END DAC3_Init 2 */

}

/**

* @brief HRTIM1 Initialization Function

* @param None

* @retval None

*/

static void MX_HRTIM1_Init(void)

{

/* USER CODE BEGIN HRTIM1_Init 0 */

/* USER CODE END HRTIM1_Init 0 */

HRTIM_TimeBaseCfgTypeDef pTimeBaseCfg = {0};

HRTIM_TimerCtlTypeDef pTimerCtl = {0};

HRTIM_TimerCfgTypeDef pTimerCfg = {0};

HRTIM_OutputCfgTypeDef pOutputCfg = {0};

/* USER CODE BEGIN HRTIM1_Init 1 */

/* USER CODE END HRTIM1_Init 1 */

hhrtim1.Instance = HRTIM1;

hhrtim1.Init.HRTIMInterruptResquests = HRTIM_IT_NONE;

hhrtim1.Init.SyncOptions = HRTIM_SYNCOPTION_NONE;

if (HAL_HRTIM_Init(&hhrtim1) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_DLLCalibrationStart(&hhrtim1, HRTIM_CALIBRATIONRATE_3) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_PollForDLLCalibration(&hhrtim1, 10) != HAL_OK)

{

Error_Handler();

}

pTimeBaseCfg.Period = 0xFFDF;

pTimeBaseCfg.RepetitionCounter = 0x00;

pTimeBaseCfg.PrescalerRatio = HRTIM_PRESCALERRATIO_MUL32;

pTimeBaseCfg.Mode = HRTIM_MODE_CONTINUOUS;

if (HAL_HRTIM_TimeBaseConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_A, &pTimeBaseCfg) != HAL_OK)

{

Error_Handler();

}

pTimerCtl.UpDownMode = HRTIM_TIMERUPDOWNMODE_UP;

pTimerCtl.DualChannelDacEnable = HRTIM_TIMER_DCDE_DISABLED;

if (HAL_HRTIM_WaveformTimerControl(&hhrtim1, HRTIM_TIMERINDEX_TIMER_A, &pTimerCtl) != HAL_OK)

{

Error_Handler();

}

pTimerCfg.InterruptRequests = HRTIM_TIM_IT_NONE;

pTimerCfg.DMARequests = HRTIM_TIM_DMA_NONE;

pTimerCfg.DMASrcAddress = 0x0000;

pTimerCfg.DMADstAddress = 0x0000;

pTimerCfg.DMASize = 0x1;

pTimerCfg.HalfModeEnable = HRTIM_HALFMODE_DISABLED;

pTimerCfg.InterleavedMode = HRTIM_INTERLEAVED_MODE_DISABLED;

pTimerCfg.StartOnSync = HRTIM_SYNCSTART_DISABLED;

pTimerCfg.ResetOnSync = HRTIM_SYNCRESET_DISABLED;

pTimerCfg.DACSynchro = HRTIM_DACSYNC_NONE;

pTimerCfg.PreloadEnable = HRTIM_PRELOAD_DISABLED;

pTimerCfg.UpdateGating = HRTIM_UPDATEGATING_INDEPENDENT;

pTimerCfg.BurstMode = HRTIM_TIMERBURSTMODE_MAINTAINCLOCK;

pTimerCfg.RepetitionUpdate = HRTIM_UPDATEONREPETITION_DISABLED;

pTimerCfg.PushPull = HRTIM_TIMPUSHPULLMODE_DISABLED;

pTimerCfg.FaultEnable = HRTIM_TIMFAULTENABLE_NONE;

pTimerCfg.FaultLock = HRTIM_TIMFAULTLOCK_READWRITE;

pTimerCfg.DeadTimeInsertion = HRTIM_TIMDEADTIMEINSERTION_DISABLED;

pTimerCfg.DelayedProtectionMode = HRTIM_TIMER_A_B_C_DELAYEDPROTECTION_DISABLED;

pTimerCfg.UpdateTrigger = HRTIM_TIMUPDATETRIGGER_NONE;

pTimerCfg.ResetTrigger = HRTIM_TIMRESETTRIGGER_NONE;

pTimerCfg.ResetUpdate = HRTIM_TIMUPDATEONRESET_DISABLED;

pTimerCfg.ReSyncUpdate = HRTIM_TIMERESYNC_UPDATE_UNCONDITIONAL;

if (HAL_HRTIM_WaveformTimerConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_A, &pTimerCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformTimerConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_C, &pTimerCfg) != HAL_OK)

{

Error_Handler();

}

pTimerCfg.DelayedProtectionMode = HRTIM_TIMER_D_E_DELAYEDPROTECTION_DISABLED;

if (HAL_HRTIM_WaveformTimerConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_D, &pTimerCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformTimerConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_E, &pTimerCfg) != HAL_OK)

{

Error_Handler();

}

pTimerCfg.DelayedProtectionMode = HRTIM_TIMER_F_DELAYEDPROTECTION_DISABLED;

if (HAL_HRTIM_WaveformTimerConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_F, &pTimerCfg) != HAL_OK)

{

Error_Handler();

}

pOutputCfg.Polarity = HRTIM_OUTPUTPOLARITY_HIGH;

pOutputCfg.SetSource = HRTIM_OUTPUTSET_NONE;

pOutputCfg.ResetSource = HRTIM_OUTPUTRESET_NONE;

pOutputCfg.IdleMode = HRTIM_OUTPUTIDLEMODE_NONE;

pOutputCfg.IdleLevel = HRTIM_OUTPUTIDLELEVEL_INACTIVE;

pOutputCfg.FaultLevel = HRTIM_OUTPUTFAULTLEVEL_NONE;

pOutputCfg.ChopperModeEnable = HRTIM_OUTPUTCHOPPERMODE_DISABLED;

pOutputCfg.BurstModeEntryDelayed = HRTIM_OUTPUTBURSTMODEENTRY_REGULAR;

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_A, HRTIM_OUTPUT_TA1, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_C, HRTIM_OUTPUT_TC1, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_D, HRTIM_OUTPUT_TD1, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_E, HRTIM_OUTPUT_TE1, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_F, HRTIM_OUTPUT_TF1, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_TimeBaseConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_C, &pTimeBaseCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformTimerControl(&hhrtim1, HRTIM_TIMERINDEX_TIMER_C, &pTimerCtl) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_C, HRTIM_OUTPUT_TC2, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_D, HRTIM_OUTPUT_TD2, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformOutputConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_F, HRTIM_OUTPUT_TF2, &pOutputCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_TimeBaseConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_D, &pTimeBaseCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformTimerControl(&hhrtim1, HRTIM_TIMERINDEX_TIMER_D, &pTimerCtl) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_TimeBaseConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_E, &pTimeBaseCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformTimerControl(&hhrtim1, HRTIM_TIMERINDEX_TIMER_E, &pTimerCtl) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_TimeBaseConfig(&hhrtim1, HRTIM_TIMERINDEX_TIMER_F, &pTimeBaseCfg) != HAL_OK)

{

Error_Handler();

}

if (HAL_HRTIM_WaveformTimerControl(&hhrtim1, HRTIM_TIMERINDEX_TIMER_F, &pTimerCtl) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN HRTIM1_Init 2 */

/* USER CODE END HRTIM1_Init 2 */

HAL_HRTIM_MspPostInit(&hhrtim1);

}

/**

* @brief TIM3 Initialization Function

* @param None

* @retval None

*/

static void MX_TIM3_Init(void)

{

/* USER CODE BEGIN TIM3_Init 0 */

/* USER CODE END TIM3_Init 0 */

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_OC_InitTypeDef sConfigOC = {0};

/* USER CODE BEGIN TIM3_Init 1 */

/* USER CODE END TIM3_Init 1 */

htim3.Instance = TIM3;

htim3.Init.Prescaler = 0;

htim3.Init.CounterMode = TIM_COUNTERMODE_UP;

htim3.Init.Period = 65535;

htim3.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim3.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_PWM_Init(&htim3) != HAL_OK)

{

Error_Handler();

}

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim3, &sMasterConfig) != HAL_OK)

{

Error_Handler();

}

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 0;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

if (HAL_TIM_PWM_ConfigChannel(&htim3, &sConfigOC, TIM_CHANNEL_4) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN TIM3_Init 2 */

/* USER CODE END TIM3_Init 2 */

HAL_TIM_MspPostInit(&htim3);

}

/**

* @brief UCPD1 Initialization Function

* @param None

* @retval None

*/

static void MX_UCPD1_Init(void)

{

/* USER CODE BEGIN UCPD1_Init 0 */

/* USER CODE END UCPD1_Init 0 */

LL_GPIO_InitTypeDef GPIO_InitStruct = {0};

/* Peripheral clock enable */

LL_APB1_GRP2_EnableClock(LL_APB1_GRP2_PERIPH_UCPD1);

LL_AHB2_GRP1_EnableClock(LL_AHB2_GRP1_PERIPH_GPIOB);

/**UCPD1 GPIO Configuration

PB4   ------> UCPD1_CC2

PB6   ------> UCPD1_CC1

*/

GPIO_InitStruct.Pin = LL_GPIO_PIN_4;

GPIO_InitStruct.Mode = LL_GPIO_MODE_ANALOG;

GPIO_InitStruct.Pull = LL_GPIO_PULL_NO;

LL_GPIO_Init(GPIOB, &GPIO_InitStruct);

GPIO_InitStruct.Pin = LL_GPIO_PIN_6;

GPIO_InitStruct.Mode = LL_GPIO_MODE_ANALOG;

GPIO_InitStruct.Pull = LL_GPIO_PULL_NO;

LL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/* USER CODE BEGIN UCPD1_Init 1 */

/* USER CODE END UCPD1_Init 1 */

/* USER CODE BEGIN UCPD1_Init 2 */

/* USER CODE END UCPD1_Init 2 */

}

/**

* @brief USART3 Initialization Function

* @param None

* @retval None

*/

static void MX_USART3_UART_Init(void)

{

/* USER CODE BEGIN USART3_Init 0 */

/* USER CODE END USART3_Init 0 */

/* USER CODE BEGIN USART3_Init 1 */

/* USER CODE END USART3_Init 1 */

huart3.Instance = USART3;

huart3.Init.BaudRate = 115200;

huart3.Init.WordLength = UART_WORDLENGTH_8B;

huart3.Init.StopBits = UART_STOPBITS_1;

huart3.Init.Parity = UART_PARITY_NONE;

huart3.Init.Mode = UART_MODE_TX_RX;

huart3.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart3.Init.OverSampling = UART_OVERSAMPLING_16;

huart3.Init.OneBitSampling = UART_ONE_BIT_SAMPLE_DISABLE;

huart3.Init.ClockPrescaler = UART_PRESCALER_DIV4;

huart3.AdvancedInit.AdvFeatureInit = UART_ADVFEATURE_NO_INIT;

if (HAL_UART_Init(&huart3) != HAL_OK)

{

Error_Handler();

}

if (HAL_UARTEx_SetTxFifoThreshold(&huart3, UART_TXFIFO_THRESHOLD_1_8) != HAL_OK)

{

Error_Handler();

}

if (HAL_UARTEx_SetRxFifoThreshold(&huart3, UART_RXFIFO_THRESHOLD_1_8) != HAL_OK)

{

Error_Handler();

}

if (HAL_UARTEx_DisableFifoMode(&huart3) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN USART3_Init 2 */

/* USER CODE END USART3_Init 2 */

}

/**

* @brief USB Initialization Function

* @param None

* @retval None

*/

static void MX_USB_PCD_Init(void)

{

/* USER CODE BEGIN USB_Init 0 */

/* USER CODE END USB_Init 0 */

/* USER CODE BEGIN USB_Init 1 */

/* USER CODE END USB_Init 1 */

hpcd_USB_FS.Instance = USB;

hpcd_USB_FS.Init.dev_endpoints = 8;

hpcd_USB_FS.Init.speed = PCD_SPEED_FULL;

hpcd_USB_FS.Init.phy_itface = PCD_PHY_EMBEDDED;

hpcd_USB_FS.Init.Sof_enable = DISABLE;

hpcd_USB_FS.Init.low_power_enable = DISABLE;

hpcd_USB_FS.Init.lpm_enable = DISABLE;

hpcd_USB_FS.Init.battery_charging_enable = DISABLE;

if (HAL_PCD_Init(&hpcd_USB_FS) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN USB_Init 2 */

/* USER CODE END USB_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* USER CODE BEGIN MX_GPIO_Init_1 */

/* USER CODE END MX_GPIO_Init_1 */

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOF_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOC, BUCKBOOST_LOAD_1_Pin|BUCKBOOST_LOAD_2_Pin|GPIO_PIN_0|BUCKBOOST_USBPD_EN_Pin

|USBPD_1A_PROTECT_Pin, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_0|GPIO_PIN_1, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_4|GPIO_PIN_5|LD2_Pin, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOB, LD3_Pin|LD5_Pin|LD4_Pin|GPIO_PIN_8

|GPIO_PIN_9, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(USBPD_550mA_PROTECT_GPIO_Port, USBPD_550mA_PROTECT_Pin, GPIO_PIN_RESET);

/*Configure GPIO pins : JOYSTICK_SEL_Pin JOYSTICK_LEFT_Pin JOYSTICK_DOWN_Pin */

GPIO_InitStruct.Pin = JOYSTICK_SEL_Pin|JOYSTICK_LEFT_Pin|JOYSTICK_DOWN_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_IT_RISING;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pins : BUCKBOOST_LOAD_1_Pin BUCKBOOST_LOAD_2_Pin PC0 BUCKBOOST_USBPD_EN_Pin

USBPD_1A_PROTECT_Pin */

GPIO_InitStruct.Pin = BUCKBOOST_LOAD_1_Pin|BUCKBOOST_LOAD_2_Pin|GPIO_PIN_0|BUCKBOOST_USBPD_EN_Pin

|USBPD_1A_PROTECT_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pins : PF0 PF1 */

GPIO_InitStruct.Pin = GPIO_PIN_0|GPIO_PIN_1;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOF, &GPIO_InitStruct);

/*Configure GPIO pins : PA4 PA5 LD2_Pin */

GPIO_InitStruct.Pin = GPIO_PIN_4|GPIO_PIN_5|LD2_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

/*Configure GPIO pins : LD3_Pin LD5_Pin LD4_Pin PB8

PB9 */

GPIO_InitStruct.Pin = LD3_Pin|LD5_Pin|LD4_Pin|GPIO_PIN_8

|GPIO_PIN_9;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/*Configure GPIO pins : JOYSTICK_RIGHT_Pin JOYSTICK_UP_Pin */

GPIO_InitStruct.Pin = JOYSTICK_RIGHT_Pin|JOYSTICK_UP_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_IT_RISING;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/*Configure GPIO pin : USBPD_550mA_PROTECT_Pin */

GPIO_InitStruct.Pin = USBPD_550mA_PROTECT_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(USBPD_550mA_PROTECT_GPIO_Port, &GPIO_InitStruct);

/* EXTI interrupt init*/

HAL_NVIC_SetPriority(EXTI2_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI2_IRQn);

HAL_NVIC_SetPriority(EXTI4_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI4_IRQn);

HAL_NVIC_SetPriority(EXTI9_5_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI9_5_IRQn);

HAL_NVIC_SetPriority(EXTI15_10_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI15_10_IRQn);

/* USER CODE BEGIN MX_GPIO_Init_2 */

/* USER CODE END MX_GPIO_Init_2 */

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */